- ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Medical Drugs, Equipment’s and Consumables	p 1 /…

Publication reference: 024/CUAMM/ETH/2023

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

LOT 1–Supply of Medical Drugs for Debrebrhan, Amhara Region – Unit price

| 1. Item number | 2. Specifications required | 3. Specifications offered TO BE FILLED BY THE SUPPLIER | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
|  | Cephalexin Specifications: 250mg/5ml, Oral Suspension Unit: Bottle of 100ml Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Paracetamol Specifications: 120mg/5ml – Syrup Unit: Bottle of 60ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Cefixime Specifications: 100mg/5ml, Oral Suspension Unit: Bottle of 100ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Cefixime Specifications: 400mg- Capsul- 7*2 Unit: Pk of 7x2 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Amlodipine Specifications: 10mg tablet Unit: PK of 10x10 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Atorvastatin Specifications: 40mg Tablet Unit: PK of 10x3 Quantity:100 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Amoxicillin + Clavulanic Acid Specifications: 125mg + 31.25mg)/5ml - Oral Suspension Unit: Bottle of 100ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Amoxicillin Specifications: 125mg/5ml 0ral Suspension Unit: Bottle of 100ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Omeprazole Specifications: 20mg Capsule Unit: PK of 10 x 10 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Cimetidine Specifications: 200mg/ml in 2ml ampoule, Injection Unit: Bottle of 10 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Amoxicillin Specifications: 500mg Capsule Unit: Pk of 10 x 10 Quantity:200 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Diclofenac Specifications: 50mg tablet Unit: Pk of 10 x 10 strip Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Cephalexin Specifications: 125mg/5ml – Oral Suspension Unit: Bottle of 100ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Oxytocin Specifications: 10 Units/ml in 1ml Ampoule Injection Unit: ampoule of 10 x 10 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Cat Gut Chromic Specifications: Gauge 2.0(4/0) 75cm on 15mm 1/2 Circle Reverse Cutting Needle Unit: Dozen Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Amoxicillin Specifications: 250mg/5ml - Oral Suspension Unit: Bottle of 100ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Azithromycin Specifications: 500mg – tablet Unit: Pk of 10 x 3 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Oral Rehydration Salt Specifications: 20.7g for 1 Littre Preparation- Powder Unit: Sachet Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Amoxicillin + Clavulanic Acid Specifications: 500mg + 125mg -Tablet ( Film-coated) Unit: Pk of 5 x 3 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Ketoconazole Specifications: 200mg Tablet Unit: Pk of 10 x 10 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Amoxicillin Specifications: 500mg Capsule Unit: Pk of 10 x 100 Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Hydrocortisone acetate 1% Specifications: Topical Ointment – 15gm Unit: Tube of 15gm Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Metronidazole Specifications: 125mg/5ml –Oral suspension Unit: Bottle of 100ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Cephalexin Specifications: 250mg/5ml Oral Suspension Unit: Bottle of 150ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Paracetamol Specifications: 120mg/5ml – Syrup Unit: Bottle of 60ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Cefixime Specifications: 100mg/5ml – Oral Suspension Unit: Bottle of 150ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Cefixime Specifications: 400mg- Capsul Unit: Pk of 7 x 2ml Expire: More than 12 months Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |

LOT 2–Supply of Medical Equipment for Debrebrhan, Amhara Region - Unit price

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
|  | Glucometer Technical Specifications: - Glucometer with test strips and lancets - Glucose meter: For glucose measurement, a hand-held device, automatic - Zero setting between measurements - Sample size of 5-10 uL (capillary, venous or arterial whole blood) - Reading time: 10sec-5min supplied with lancets and test strips specified below - Unit: Each - Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Delivery Bed General Description: Bed, labour delivery, with accessories Technical Specifications: • Bed, labour and delivery, 2 sections. • All sections fit with padded mattress, detachable from bed for easy cleaning. • Mattress covers removable via side zipper. Body section: • Mounted on 4 sturdy supports, finished with rubber feet. • Knee crutch holders welded to the frame of the bed. • Crutches are height and width adjustable, set with sturdy clamps with heavy knob. • Leg section • Mounted on swivel castors, of which two with brakes. • This section recesses entirely under body section. • When fully extended, both sections align to perfectly flat surface. Materials: • High resistance to corrosion (tropical environment). • Frame: epoxy coated tubular steel. • Sliders/fixtures for the knee crutches: tubular steel, welded to the bed frame. • Recession tracks smoothly finished for easy sliding. • Mattress: high-density polyurethane foam, density approx. 30 kg/m3. • Cover: plastic, flexible highly tear resistant, anti-static, flame retardant, disinfectant- and liquid proof, washable. Dimensions: • Body section: approx. 1000 x 900 x 750 mm (l x w x h). • Leg section: approx. 900 x 850 x 750 mm (l x w x h). • Frame, diameter: approx. 40 mm. • Swivel castors, diameter: approx. 120mm. • Mattress: approx. 100 mm (h) Carrying capacity: approx. 150kg. Supplied with: • 1 x set of tools required for assembly. • 2 x leg holders with canvas straps, adjustable height and width. • 2 x knee crutches, adjustable height and width. • 1 x set fitting mattresses, body and leg section. • List of parts. Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Sphygmomanometer (Digital) Technical Specifications: - Adult; with power display 4xAAA Batteries or Micro-USB cable, LCD display 60x64mm, Cuff pressure range:0~299 mmHg - Blood pressure measurement range: SYS 60-230 mmHg, - DIA 40-130 mmHg - Pulse measurement range: 40~199 beat/min, Cuff type 22-42cm or 22-32cm (optional), Dimensions 100x129x45mm, Type BF applied part and IP21 Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Sphygmomanometer ( Anaroid) Technical Specifications: - Adult; Pressure Range 0- 300mm aneroid model - Complete with adult velcro cuff bulb and value in vinyl case ( typical dimensions (0.15x0.15x0.04 cm) - Weight 0.30 kg Unit: Each Delivery: Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Stethoscope Technical Specifications: - has stainless steel/ chromed brass - Double-bell chest piece (dual-use adult and paediatric auscultation). - Adult diaphragm: approx 43 mm. - Pediatric diaphragm: approx 28 mm. - Bi-aural unit,Double stent chest piece.,Plain spring non-folding frame, Plastic ear tips, Vinyl stethoscope tubing, Combined bell and diaphragm sprague type. - Sensitivity, 50 to 500 Hz: 3.2 dB (cardiology). - Sensitivity, 600 to 1500 Hz: 8.1 dB (pneumology). - Single tube in treated rubber, lumen diameter: approx 10 mm. - Y-part is reinforced with stainless steel spring attached to the rigid ear tubes. - Spring is treated to give long lasting rebound and comfort. - Ear tubes are made of stainless steel or chromed brass. - Ear-pieces are made of plastic and are removable. - Total length, approx 70 cm. Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Otoscope, instruments GENERAL DESCRIPTION: Consultation instrument for ENT TECHNICAL CHARACTERISTICS Pneumatic consultation otoscope - Metallic construction - Integrated Halogen bulb 3.50 V - Fiber optic transmission - Ear insufflating system - Rotating lens with 10D minimum magnification and sealing system Otoscope handle362 - Metallic construction - Rheostat light intensity control - Rechargeable battery included - Convertible to normal batteries Otoscope accessories - Set of reusable sterilizable specula - Battery charger 220 V ac / 50 Hz - Case / bag for storage and transport Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Reflex hammer General Description: Hammer, reflex testing, Features: - Taylor type or similar - Percussion for Neurology examination Complete with; Two rubber heads, small and large Metal handle (approx.) 18 cm. Brush screw into the end of the handle Needle screws at the top of the handle Technical Specifications: • Hammer, reflex testing, Taylor type, regular size, approx: 18 cm. • Solid metal handle, chrome plated, solid rubber head. Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach the catalogue |  |  |
|  | Tuning fork Technical Specifications: - Used to assess hearing through vibration - Made of steel Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach the catalogue |  |  |
|  | Examination bed; Technical Specifications: Couch, examination General Description: Table, examination in 2 sections. Technical Specifications: • Mounted on 4 sturdy supports, finished with rubber feet, of which one height adjustable. • Both sections fit with thick upholstery. • Backrest adjustable via secured pawl and gear ratchet, safe for patient and operator. • When fully extended, both sections align to perfectly flat surface. Materials: • High resistance to corrosion (tropical environment). • Frame: epoxy coated tubular steel. • Upholstery: high-density polyurethane foam, density approx. 30 kg/m3. • Cover: plastic, flexible highly tear resistant, anti-static, flame retardant, disinfectant- and liquid proof, washable. Dimensions: • All sections extended: approx. 1800 x 550 x 750 mm (l x w x h). • Frame, diameter: approx. 30 mm. • Upholstery: approx. 50 mm (h) • Carrying capacity: approx. 150kg. • Standard examination table for health care facilities. Must be cleaned after each use. Unit: pack Delivery: CUAMM Addis Ababa | Please provide delivery time and attach the catalogue |  |  |
|  | Adult Weight scale/digital General Description: Digital adult weighing scale Technical Specifications: • Easy to read large digital display1½ inch LCD display • Weighing range: 0 up to 150 kg • Scale accurately reads weight in 500 g increments • Extra wide base with non-slip foot guides • Easy cleaning and disinfection • Springless technology, electronic measuring. • Turns on instantly when you step on • Recalibrates automatically to zero when you step off • Operates on one 9-volt battery • Low battery indicator Material: Heavy duty plastic or synthetics Packaging and labelling: Primary packaging: Unit of use One (1) digital adult scale in box, with manufacturer's instruction for use. Labelling on the packaging unit: Labelling to be the same as primary packaging. Accessories/Spare parts/Consumables: 9V batteries Weight/Volume/Dimensions: • estimated weight: 2.5kg • estimated volume: 3 cdm • Instructions for use: Weighing scale to be used in context of adult medical examination at different in- and outpatient department of a hospital. Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach the catalogue |  |  |
|  | Baby Weight scale Description: Weighing, Scale, electronic for infants (Baby scale) Features: . Full electronic . Portable . Capacity: 20kg395 . Division: 10g Construction: Durable, unbreakable sturdy material Display: . Digital readings . LCD screen . Large figures . In Kilograms Control system: . Front panel . Button (s) control for on-tare-off functions . Automatic: Zero Setting . Weight locks function (preferable) Power supply: . Battery operated by using 9 V., battery (to be included) . Automatic off switch to save battery life Baby tray: . Detachable . Made of unbreakable material . Easy to clean and disinfect . Anti-tilt . Side with safe edges ends and of suitable heights - Dimensions: 500 x 250 x 100 mm (L x W x H) (approx.) Safety: All safety measures to be considered for the baby & staff Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach the catalogue |  |  |
|  | Speculum-Vaginal, Medium Set Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach the catalogue |  |  |
|  | Digital Thermometer Description: Thermometer, clinical, digital, 32 - 430C Technical Specifications: • Digital thermometer Celsius scale with switch to Fahrenheit • Safe to use, atraumatic, no glass, no mercury • Measurement range: 32°C to 43°C • Accurate measurement: +/- 0.1°C between 35°C to 41°C • Liquid crystal display, easy to read • Beep sound and switch off • Water proof for ease of cleaning • Battery powered Unit: Each Delivery: CUAMM Addis Ababa | Please provide delivery time and attach the catalogue |  |  |

LOT 2–Supply of Medical Consumables for Debrebrhan, Amhara Region

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
|  | Measuring tape – Each Quantity:100 Unit: Each Delivery: CUAMM Addis Ababa |  |  |  |
|  | Vacutainer Blood Collection Tube - with EDTA anticoagulant purple top 4ml Plastic Quantity:100 Unit: Each Delivery: CUAMM Addis Ababa |  |  |  |
|  | Glucose test strips - 50 tests Quantity:50 Unit: Box of 50 test Delivery: CUAMM Addis Ababa |  |  |  |
|  | Glove Surgical Latex Sterile Powder Free - No. 7 – 50 pairs Quantity:30 Unit: Pair Box of 50 pair Delivery: CUAMM Addis Ababa |  |  |  |
|  | Glove Examination Latex Non Powdered – Medium – 100 Quantity:30 Unit: Box of 100 Delivery: CUAMM Addis Ababa |  |  |  |
|  | Alcohol Based Hand Rub (Hand Sanitizer) -500ml Quantity:50 Unit: Bottle of 500ml Delivery: CUAMM Addis Ababa |  |  |  |
|  | Face Mask Surgical – Surgical- 50 Quantity:50 Unit: Box of 50 Delivery: CUAMM Addis Ababa |  |  |  |

LOT 4–Supply of Medical Drugs for Dassenech Woreda, Health office South OMO.

| 1. Item number | 2. Specifications required | 3. Specifications offered TO BE FILLED BY THE SUPPLIER | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
|  | Adrenalin Specifications: (Epinephrine) 0.1% in ampule – injection Unit: amp Quantity:150 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Albendazole Specifications: 100mg/5ml - Oral Suspension 20ml Unit: Bottle Quantity:250 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Ampicillin Specifications: 250mg – Capsule Unit: 100x10 Quantity:25 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Benzoic Specifications: acid+salicylic acid - (6% +3%) - Topical ointment, 20mg Unit: Quantity:50 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Bisocodyle Specifications: 5mg tab Unit: 10 x 10 Quantity:10 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Diclofenac Specifications: 50 mg po Unit: 10 x 10 Quantity:100 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Erythromycin Specifications: Stearate - 500mg – Tablet Unit: 10 x 9 Quantity:100 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Ferrous Sulfate + Folic Acid Specifications: (150mg +0.5mg) - Tablet (Enteric Coated) Unit: pkt Quantity:100 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Ictamol Specifications: 10% skin ointment, 20mg Unit: Quantity:25 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Mebendazole Specifications: 100mg/5ml syrup,30ml Unit: Bottel Quantity:250 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Multivitamin Specifications: with Minerals –Tablet Unit: 10 x 10 Quantity:50 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Oral Rehydration Salt Specifications: 20.7g for 1 Littre Preparation Powder Unit: Sachet Quantity:4000 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Pencillin Specifications: G. crystalin 10ml/IU Unit: 100x1 Quantity:10 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Procane pencillin Specifications: fortified inj Unit: 100x1 Quantity:20 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Vitamin B Specifications: Complex Preparations – Tablet Unit: 10x10 Quantity:50 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |
|  | Zinc Oxide Specifications: 15% Cream, 20mg Unit: Quantity:100 Expire: 12 months or longer Delivery: CUAMM Addis Ababa | Specification offered: Unit: Expires Date: Delivery time: (TO BE FILLED) |  |  |

LOT 5–Supply of Medical Equipment for Dassenech Woreda, Health office South OMO.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
|  | Sucker (bird type) Unit: Each Quantity:10 Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Multi - functioning Delivery coach Unit: Each Quantity:6 Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Delivery set; with instruments (2 stainless steel kidney dishes, 25 cm 1 st.st. Triangular dish (placenta dish) 1 st.st. Bowl, 6" 1 Mayo scissors, straight, 15 cm 2 Spencer Well's artery forceps, straight, 20 cm 1 Sponge holder, 25 cm 1 Cord scissors 1 Episiotomy scissors 2 Female catheter Unit: Each Quantity:8 Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Sphygmomanometer Digital, Adult; with power display 4xAAA Batteries or Micro-USB cable, LCD display 60x64mm, Cuff pressure range:0~299 mmHg Blood pressure measurement range: SYS 60-230 mmHg, DIA 40-130 mmHg Pulse measurement range: 40~199 beat/min, Cuff type 22-42cm or 22-32cm (optional), Dimensions 100x129x45mm, Type BF applied part and IP21 Unit: Each Quantity:8 Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Examination bed; Size: 1920x740x380 mm Back section: 695 × 620 mm Backrest and Knee rest movement on ratchet Framework made of Stainless Steel. Mounted on protective Stumps. Freight saving knock-down construction Pretreated and powder coated Unit: Each Quantity:5 Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Puls oxymetery Unit: Each Quantity:5 Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Ambo bag for adult Unit: Each Quantity:5 Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |
|  | Ambo bag for neonate Unit: Each Quantity:5 Delivery: CUAMM Addis Ababa | Please provide delivery time and attach catalogue |  |  |